ESCUELA COLOMBIANA DE INGENIERÍA JULIO GARAVITO INGENIERÍA DE SISTEMAS

REDES DE COMPUTADORES

Ing. Claudia P Santiago

Resuelva los siguientes ejercicios

- La compañía de productos naturales HB abrió operaciones en Colombia hace dos meses y construyó una gran planta de producción en Bogotá. El gerente general de la empresa ha decidido interconectar todos los computadores de la empresa y permitir que todos ellos tengan acceso a Internet, excepto los de financiera (por seguridad). Como el gerente no sabe mucho del asunto, habló con un amigo que le recomendó dividir la red por áreas así: Producción, Ventas, Publicidad, Investigación y Financiero (que no tiene acceso a Internet).

En cuanto a equipos, en la actualidad se tienen 25 en el área de producción, 20 en el área de ventas, 5 en el área de publicidad, 8 en el área de Investigación y 35 en el área de financiera y se espera un crecimiento máximo de 5 equipos por área.

Después de haber hecho toda esta investigación, el amigo del gerente le recomienda usar TCP/IP en toda la red y contratarlo para que usted defina la clase de direcciones IP que debe adquirir (lo más eficiente posible) y le entregue la configuración IP de cada área de la empresa (número de direcciones máximas en cada red, máscaras y direcciones de broadcast y red). Recuerde que todos los equipos deben tener dirección IP pero la red financiera es privada., para ésta debe hacerse uso de un rango de direcciones privado.

Problem Analysis:

Current Equipment:

- Production: 25 computers

- Sales: 20 computers

- Advertising: 5 computers

- Research: 8 computers

- Finance: 35 computers

- Total: 93 computers

Growth: Maximum 5 computers per area

Future Equipment:

- Production: 25 + 5 = 30

- Sales: 20 + 5 = 25

- Advertising: 5 + 5 = 10

- Research: 8 + 5 = 13

- Finance: 35 + 5 = 40

Requirements:

- 5 separate networks (subnets)

- Finance must use private IP range

- Most efficient IP addressing

- Need: Network address, Broadcast, Mask, Max stations

Solution Strategy:

Step 1: Choose the most efficient private IP block

Since we need:

- 5 subnets

- Maximum subnet needs 40 hosts (Finance)

Best choice: Use 192.168.0.0/24 (Class C private range)

For 40 hosts: Need 2^6 - 2 = 62 hosts → Use /26 (gives 62 usable hosts)

For 5 subnets with /26:

- Each /26 subnet = 64 addresses

- Can fit 4 subnets in one /24

- Need to use 192.168.0.0/24 and 192.168.1.0/24

Step 2: Assign subnets (ordered by size)

Using VLSM (Variable Length Subnet Mask) for efficiency:

- Finance (40 hosts) → /26 (62 hosts)

- Production (30 hosts) → /26 (62 hosts)

- Sales (25 hosts) → /27 (30 hosts)

- Research (13 hosts) → /28 (14 hosts)

- Advertising (10 hosts) → /28 (14 hosts)

SOLUTION TABLE:

Detailed Breakdown:

1. Producción (30 computers needed)

- Network: 192.168.0.0/26

- Broadcast: 192.168.0.63

- Mask: 255.255.255.192

- Range: 192.168.0.1 - 192.168.0.62

- Max stations: 62

- Binary subnet part: 00000000 (last octet network = 0)

2. Ventas (25 computers needed)

- Network: 192.168.0.64/27

- Broadcast: 192.168.0.95

- Mask: 255.255.255.224

- Range: 192.168.0.65 - 192.168.0.94

- Max stations: 30

- Binary subnet part: 01000000 (last octet network = 64)

3. Publicidad (10 computers needed)

- Network: 192.168.0.96/28

- Broadcast: 192.168.0.111

- Mask: 255.255.255.240

- Range: 192.168.0.97 - 192.168.0.110

- Max stations: 14

- Binary subnet part: 01100000 (last octet network = 96)

4. Investigación (13 computers needed)

- Network: 192.168.0.112/28

- Broadcast: 192.168.0.127

- Mask: 255.255.255.240

- Range: 192.168.0.113 - 192.168.0.126

- Max stations: 14

- Binary subnet part: 01110000 (last octet network = 112)

5. Financiero (40 computers needed) - PRIVATE RANGE

- Network: 10.0.0.0/26

- Broadcast: 10.0.0.63

- Mask: 255.255.255.192

- Range: 10.0.0.1 - 10.0.0.62

- Max stations: 62

- Binary subnet part: 00000000 (last octet network = 0)

Verification:

✅ All areas have enough capacity:

- Production: 30 needed, 62 available ✓

- Sales: 25 needed, 30 available ✓

- Advertising: 10 needed, 14 available ✓

- Research: 13 needed, 14 available ✓

- Finance: 40 needed, 62 available ✓

✅ Finance uses separate private range (10.0.0.0/8)

✅ Efficient use of IP space using VLSM

✅ All subnets can communicate through routers

- Un nuevo ISP está captando clientes y sus vendedores han traído las siguientes empresas para prestarles servicio de conexión a Internet y asignación de rangos IP para sus servidores:

  - Exportadora de flores “rosita” que requiere direcciones para 16 host

  - Pastas “Magola” que necesita direcciones para 10 host

  - Supermercados “buen Servir” que necesita direcciones para 28 host

  - Autos “Super cars” que requiere direcciones para 55 host

  - Inmobiliaria “su casita” que requiere direcciones para 4 host

  - Comercializadora “Rojas&co” que requiere direcciones para 2 host

El ISP acaba de comprar un rango tipo C y quiere saber si es suficiente este rango para atender a sus clientes o debe comprar uno adicional. Si es suficiente con el rango actual, requiere saber cuál sería la distribución de subredes (dirección de red, broadcast, máscara, número de direcciones asignadas) para cada uno de ellos.

Si el rango no alcanza para atender toda la demanda, indicar la distribución de subredes que máximo se podría cubrir con el rango actual y dejar especificado el o los rangos que deben ser cubiertos con el nuevo rango a comprar.

Nota: Deje expresado los rangos de subnetting en binario y las máscaras en formato /xx En un trabajo adicional, el ISP tiene 7 nuevos clientes con requerimientos de direcciones IP así:

  - Aba publicaciones: 40 direcciones IP

  - Buques de la costa: 18 direcciones IP

  - Caritas felices: 6 direcciones IP

  - Diseño futurista: 29 direcciones IP

  - EsferosEcológicos: 12 direcciones IP

  - Fundación CG: 35 direcciones IP

  - Galvanizados Industriales: 2 direcciones IP

Part 1: Initial 6 clients with Class C range

Client requirements:

- Exportadora de flores "Rosita": 16 hosts

- Pastas "Magola": 10 hosts

- Supermercados "Buen Servir": 28 hosts

- Autos "Super cars": 55 hosts

- Inmobiliaria "Su casita": 4 hosts

- Comercializadora "Rojas&co": 2 hosts

Total: 16 + 10 + 28 + 55 + 4 + 2 = 115 hosts needed

Step 1: Check if one Class C is sufficient

Class C network: Can be any from 192.0.0.0 - 223.255.255.0

Let's use 200.100.50.0/24 as example

Available addresses in /24: 256 total, 254 usable

Required using VLSM:

- We need to subnet efficiently using Variable Length Subnet Masks

Step 2: Order clients by size (largest first for VLSM)

- Super cars: 55 hosts → need /26 (62 hosts)

- Buen Servir: 28 hosts → need /27 (30 hosts)

- Rosita: 16 hosts → need /27 (30 hosts)

- Magola: 10 hosts → need /28 (14 hosts)

- Su casita: 4 hosts → need /29 (6 hosts)

- Rojas&co: 2 hosts → need /30 (2 hosts)

PART 1 SOLUTION: Distribution with Class C (200.100.50.0/24)

| Cliente | Dirección de red | Broadcast | Máscara | Máscara (/xx) | IPs asignadas | Subnetting binario |
| --- | --- | --- | --- | --- | --- | --- |
| Super cars | 200.100.50.0 | 200.100.50.63 | 255.255.255.192 | /26 | 62 | 200.100.50.00000000 |
| Buen Servir | 200.100.50.64 | 200.100.50.95 | 255.255.255.224 | /27 | 30 | 200.100.50.01000000 |
| Rosita | 200.100.50.96 | 200.100.50.127 | 255.255.255.224 | /27 | 30 | 200.100.50.01100000 |
| Magola | 200.100.50.128 | 200.100.50.143 | 255.255.255.240 | /28 | 14 | 200.100.50.10000000 |
| Su casita | 200.100.50.144 | 200.100.50.151 | 255.255.255.248 | /29 | 6 | 200.100.50.10010000 |
| Rojas&co | 200.100.50.152 | 200.100.50.155 | 255.255.255.252 | /30 | 2 | 200.100.50.10011000 |

Used addresses: 0-155 (156 addresses)

Remaining: 156-255 (100 addresses available)

Answer to Part 1:

✅ YES, one Class C range IS sufficient!

The current Class C range can accommodate all 6 clients with room to spare (100 addresses remaining).

PART 2: Additional 7 new clients

New requirements:

- Aba publicaciones: 40 IPs

- Buques de la costa: 18 IPs

- Caritas felices: 6 IPs

- Diseño futurista: 29 IPs

- EsferosEcológicos: 12 IPs

- Fundación CG: 35 IPs

- Galvanizados Industriales: 2 IPs

Total new: 40 + 18 + 6 + 29 + 12 + 35 + 2 = 142 hosts

Step 3: Check remaining space

Remaining from first Class C: 100 addresses Needed for new clients: 142 addresses (considering subnet overhead)

Let's calculate exact needs with VLSM:

- Aba publicaciones: 40 → /26 (62 hosts)

- Fundación CG: 35 → /26 (62 hosts)

- Diseño futurista: 29 → /27 (30 hosts)

- Buques: 18 → /27 (30 hosts)

- Esferos: 12 → /28 (14 hosts)

- Caritas: 6 → /29 (6 hosts)

- Galvanizados: 2 → /30 (2 hosts)

Total addresses needed: 62 + 62 + 30 + 30 + 14 + 6 + 2 = 206 addresses

Available: 100 addresses

❌ NOT ENOUGH! Need additional range.

PART 2 SOLUTION:

Maximum coverage with current range (200.100.50.156-255):

| Cliente | Dirección de red | Broadcast | Máscara | /xx | IPs | Subnetting binario |
| --- | --- | --- | --- | --- | --- | --- |
| Aba publicaciones | 200.100.50.192 | 200.100.50.255 | 255.255.255.192 | /26 | 62 | 200.100.50.11000000 |

Only 1 client can fit!

New range needed (e.g., 200.100.51.0/24):

| Cliente | Dirección de red | Broadcast | Máscara | /xx | IPs | Subnetting binario |
| --- | --- | --- | --- | --- | --- | --- |
| Fundación CG | 200.100.51.0 | 200.100.51.63 | 255.255.255.192 | /26 | 62 | 200.100.51.00000000 |
| Diseño futurista | 200.100.51.64 | 200.100.51.95 | 255.255.255.224 | /27 | 30 | 200.100.51.01000000 |
| Buques costa | 200.100.51.96 | 200.100.51.127 | 255.255.255.224 | /27 | 30 | 200.100.51.01100000 |
| Esferos Ecológicos | 200.100.51.128 | 200.100.51.143 | 255.255.255.240 | /28 | 14 | 200.100.51.10000000 |
| Caritas felices | 200.100.51.144 | 200.100.51.151 | 255.255.255.248 | /29 | 6 | 200.100.51.10010000 |
| Galvanizados Ind. | 200.100.51.152 | 200.100.51.155 | 255.255.255.252 | /30 | 2 | 200.100.51.10011000 |

FINAL ANSWER:

Part 1: ✅ Sufficient with one Class C

- Current range 200.100.50.0/24 can handle all 6 initial clients

- 100 addresses remain available

Part 2: ❌ NOT sufficient - need additional range

Distribution:

- From original range (200.100.50.156-255): Aba publicaciones only

- Need NEW Class C (200.100.51.0/24): Remaining 6 clients

Ranges to purchase:

- ✅ Already have: 200.100.50.0/24

- 🆕 Must buy: 200.100.51.0/24

🎯 ISP needs to purchase one additional Class C range!

- A partir de la dirección 211.23.40.0/22, asignada por el ISP, realice la división de subredes según lo solicitado en el gráfico. Recuerde hacer la asignación de identificadores de subred en orden y lo más eficiente posible (ordenado de menor a mayor).

Problem Analysis:

Given:

- Network: 211.23.40.0/22

- Available addresses: 2^10 = 1,024 addresses (1,022 usable hosts)

Requirements from diagram:

- Switch1 network: 260 hosts

- Switch0 network: 150 hosts

- Switch2 network: 350 hosts

- Router interconnections: 3 WAN links (Router0-Router1, Router1-Router2, Router0-Router2)

Total subnets needed: 6

- 3 LAN segments

- 3 WAN links

Step 1: Order by size (VLSM - largest first)

- Switch2: 350 hosts → need 2^9 - 2 = 510 hosts → /23 (511 usable)

- Switch1: 260 hosts → need 2^9 - 2 = 510 hosts → /23 (511 usable)

- Switch0: 150 hosts → need 2^8 - 2 = 254 hosts → /24 (254 usable)

- WAN Link 1 (Router0-Router1): 2 hosts → /30

- WAN Link 2 (Router1-Router2): 2 hosts → /30

- WAN Link 3 (Router0-Router2): 2 hosts → /30

Step 2: Calculate available space

Original network: 211.23.40.0/22

- Binary: 211.23.00101000.00000000 to 211.23.00101011.11111111

- Range: 211.23.40.0 - 211.23.43.255

- Third octet range: 40-43 (binary: 101000 to 101011)

Step 3: Subnet allocation (ordered smallest to largest)

Subnet 1: Switch2 (350 hosts) - LARGEST

- Network: 211.23.40.0/23

- Mask: 255.255.254.0

- Range: 211.23.40.0 - 211.23.41.255

- First host: 211.23.40.1

- Last host: 211.23.41.254

- Broadcast: 211.23.41.255

- Usable hosts: 510

- Binary (3rd octet): 00101000 - 00101001 (40-41)

Subnet 2: Switch1 (260 hosts) - SECOND LARGEST

- Network: 211.23.42.0/23

- Mask: 255.255.254.0

- Range: 211.23.42.0 - 211.23.43.255

- First host: 211.23.42.1

- Last host: 211.23.43.254

- Broadcast: 211.23.43.255

- Usable hosts: 510

- Binary (3rd octet): 00101010 - 00101011 (42-43)

PROBLEM: We've used all /22 space (40-43)! Need to recalculate...

REVISED SOLUTION - More efficient VLSM:

Let me recalculate using the exact space more efficiently:

Subnet 1: Switch2 (350 hosts)

- Need: 2^9 = 512 addresses

- Network: 211.23.40.0/23

- Mask: 255.255.254.0

- Broadcast: 211.23.41.255

- Hosts: 510

- Binary: 211.23.00101000.00000000 to 211.23.00101001.11111111

Subnet 2: Switch1 (260 hosts)

- Need: 2^9 = 512 addresses

- Network: 211.23.42.0/23

- Mask: 255.255.254.0

- Broadcast: 211.23.43.255

- Hosts: 510

- Binary: 211.23.00101010.00000000 to 211.23.00101011.11111111

All /22 space used! For Switch0 and WAN links, we need to re-subnet differently.

CORRECTED EFFICIENT SOLUTION:

Since 350 + 260 + 150 = 760 hosts, let me use a better approach:

Subnet 1: Switch2 (350 hosts) - Use /23

- Network: 211.23.40.0/23

- Mask: 255.255.254.0

- Range: 211.23.40.1 - 211.23.41.254

- Broadcast: 211.23.41.255

- Hosts: 510

- Binary: 211.23.00101000.00000000/23

Subnet 2: Switch1 (260 hosts) - Use /24

Actually, 260 hosts needs more than 254, so needs /23

Wait, let me recalculate properly:

260 hosts → 2^9 - 2 = 510 ✓ (need /23) 350 hosts → 2^9 - 2 = 510 ✓ (need /23)

Two /23 subnets = 1024 addresses = entire /22!

We need to use smaller subnets!

FINAL OPTIMIZED SOLUTION:

Using /24 for 260 hosts won't work (only 254 hosts).

Let me use a creative approach - subnet the /22 into /23s and /24s:

COMPLETE SOLUTION TABLE:

| Subnet | Red | Dirección de red | Broadcast | Máscara | /xx | Hosts | Binary (last 2 octets) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Switch2 | 1 | 211.23.40.0 | 211.23.41.255 | 255.255.254.0 | /23 | 510 | 00101000.00000000 |
| Switch1 | 2 | 211.23.42.0 | 211.23.42.255 | 255.255.255.0 | /24 | 254 | 00101010.00000000 |
| Switch0 | 3 | 211.23.43.0 | 211.23.43.255 | 255.255.255.0 | /24 | 254 | 00101011.00000000 |
| Router0-Router1 | 4 | 211.23.42.0 | - | - | - | - | (Share Switch1 network) |
| Router1-Router2 | 5 | 211.23.42.0 | - | - | - | - | (Share Switch1 network) |
| Router0-Router2 | 6 | 211.23.42.0 | - | - | - | - | (Share Switch1 network) |

WAIT - This doesn't work for 260 hosts in /24!

CORRECT FINAL SOLUTION:

Since Switch1 needs 260 hosts and /24 only provides 254, the problem requirements exceed the /22 capacity when using standard subnetting.

Best practical solution:

| Network | Dirección de red | Broadcast | Máscara (/xx) | Hosts | Binary |
| --- | --- | --- | --- | --- | --- |
| Switch2 (350) | 211.23.40.0 | 211.23.40.255 | 255.255.255.0 (/24) | 254 ❌ Need more! |  |
| Switch1 (260) | 211.23.41.0 | 211.23.41.255 | 255.255.255.0 (/24) | 254 ❌ Need more! |  |
| Switch0 (150) | 211.23.42.0 | 211.23.42.255 | 255.255.255.0 (/24) | 254 ✓ |  |
| WAN 1 | 211.23.43.0 | 211.23.43.3 | 255.255.255.252 (/30) | 2 ✓ |  |
| WAN 2 | 211.23.43.4 | 211.23.43.7 | 255.255.255.252 (/30) | 2 ✓ |  |
| WAN 3 | 211.23.43.8 | 211.23.43.11 | 255.255.255.252 (/30) | 2 ✓ |  |

⚠️ PROBLEM: The /22 network is insufficient for the requirements (260 and 350 hosts)!

ISP needs to assign a /21 or larger network to meet all requirements.

- Si la asignación dada por ISP es 140.72.32.0/22. Realice el subnetting para las siguientes:

| RED (Ciudad) | No Hosts |
| --- | --- |
| Chía | 180 |
| Cajicá | 30 |
| Soacha | 55 |
| Madrid | 14 |
| Medellín | 43 |
| Bogotá | 130 |

Problem Analysis:

Given:

- Network: 140.72.32.0/22

- Available addresses: 2^10 = 1,024 total (1,022 usable)

Requirements:

| Ciudad | Hosts needed |
| --- | --- |
| Chía | 180 |
| Cajicá | 30 |
| Soacha | 55 |
| Madrid | 14 |
| Medellín | 43 |
| Bogotá | 130 |

Total hosts: 180 + 30 + 55 + 14 + 43 + 130 = 452 hosts

Step 1: Determine subnet sizes (VLSM)

Order by size and calculate required subnet mask:

- Chía: 180 hosts → 2^8 - 2 = 254 hosts → /24 ✓

- Bogotá: 130 hosts → 2^8 - 2 = 254 hosts → /24 ✓

- Soacha: 55 hosts → 2^6 - 2 = 62 hosts → /26 ✓

- Medellín: 43 hosts → 2^6 - 2 = 62 hosts → /26 ✓

- Cajicá: 30 hosts → 2^5 - 2 = 30 hosts → /27 ✓

- Madrid: 14 hosts → 2^4 - 2 = 14 hosts → /28 ✓

Step 2: Understand the /22 network space

Network: 140.72.32.0/22

Binary breakdown:

140.72.00100000.00000000  (140.72.32.0)

140.72.00100011.11111111  (140.72.35.255)

Range: 140.72.32.0 - 140.72.35.255

- Third octet: 32-35 (binary: 00100000 to 00100011)

Step 3: VLSM Allocation (ordered largest to smallest)

1. Chía - 180 hosts (/24)

- Network: 140.72.32.0/24

- Mask: 255.255.255.0

- First host: 140.72.32.1

- Last host: 140.72.32.254

- Broadcast: 140.72.32.255

- Usable hosts: 254

- Binary (3rd octet): 00100000

2. Bogotá - 130 hosts (/24)

- Network: 140.72.33.0/24

- Mask: 255.255.255.0

- First host: 140.72.33.1

- Last host: 140.72.33.254

- Broadcast: 140.72.33.255

- Usable hosts: 254

- Binary (3rd octet): 00100001

3. Soacha - 55 hosts (/26)

- Network: 140.72.34.0/26

- Mask: 255.255.255.192

- First host: 140.72.34.1

- Last host: 140.72.34.62

- Broadcast: 140.72.34.63

- Usable hosts: 62

- Binary (4th octet): 00000000

4. Medellín - 43 hosts (/26)

- Network: 140.72.34.64/26

- Mask: 255.255.255.192

- First host: 140.72.34.65

- Last host: 140.72.34.126

- Broadcast: 140.72.34.127

- Usable hosts: 62

- Binary (4th octet): 01000000

5. Cajicá - 30 hosts (/27)

- Network: 140.72.34.128/27

- Mask: 255.255.255.224

- First host: 140.72.34.129

- Last host: 140.72.34.158

- Broadcast: 140.72.34.159

- Usable hosts: 30

- Binary (4th octet): 10000000

6. Madrid - 14 hosts (/28)

- Network: 140.72.34.160/28

- Mask: 255.255.255.240

- First host: 140.72.34.161

- Last host: 140.72.34.174

- Broadcast: 140.72.34.175

- Usable hosts: 14

- Binary (4th octet): 10100000

COMPLETE SOLUTION TABLE:

| Ciudad | Hosts | Red (Network) | Primera IP | Última IP | Broadcast | Máscara | /n | Binary Subnetting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chía | 180 | 140.72.32.0 | 140.72.32.1 | 140.72.32.254 | 140.72.32.255 | 255.255.255.0 | /24 | 140.72.00100000.00000000 |
| Bogotá | 130 | 140.72.33.0 | 140.72.33.1 | 140.72.33.254 | 140.72.33.255 | 255.255.255.0 | /24 | 140.72.00100001.00000000 |
| Soacha | 55 | 140.72.34.0 | 140.72.34.1 | 140.72.34.62 | 140.72.34.63 | 255.255.255.192 | /26 | 140.72.00100010.00000000 |
| Medellín | 43 | 140.72.34.64 | 140.72.34.65 | 140.72.34.126 | 140.72.34.127 | 255.255.255.192 | /26 | 140.72.00100010.01000000 |
| Cajicá | 30 | 140.72.34.128 | 140.72.34.129 | 140.72.34.158 | 140.72.34.159 | 255.255.255.224 | /27 | 140.72.00100010.10000000 |
| Madrid | 14 | 140.72.34.160 | 140.72.34.161 | 140.72.34.174 | 140.72.34.175 | 255.255.255.240 | /28 | 140.72.00100010.10100000 |

Summary:

Address Space Usage:

- Used: 140.72.32.0 - 140.72.34.175

- Remaining: 140.72.34.176 - 140.72.35.255 (available for future growth)

Efficiency:

- Total required: 452 hosts

- Total allocated: 254 + 254 + 62 + 62 + 30 + 14 = 676 usable hosts

- Original /22: 1,022 usable hosts

- Utilization: ~66% (very efficient with VLSM)

Binary Subnetting Details:

Last octet binary for 140.72.34.x subnets:

Soacha:    00000000 - 00111111  (0-63)    /26

Medellín:  01000000 - 01111111  (64-127)  /26

Cajicá:    10000000 - 10011111  (128-159) /27

Madrid:    10100000 - 10101111  (160-175) /28

Verification:

✅ All cities have enough capacity:

- Chía: 180 needed, 254 available

- Bogotá: 130 needed, 254 available

- Soacha: 55 needed, 62 available

- Medellín: 43 needed, 62 available

- Cajicá: 30 needed, 30 available

- Madrid: 14 needed, 14 available

✅ No overlapping subnets ✅ Efficient use of VLSM ✅ All within 140.72.32.0/22 range

- Realice el ejercicio definido en el siguiente diagrama de red

Problem Analysis:

Network Requirements:

From the diagram, I can identify:

LAN segments:

- LAN2 (Bogotá-sur) - Switch1: 20 hosts (PC2, PC3)

- LAN1 (Bogotá-sur) - Switch0: 30 hosts (PC0, PC1)

- LAN1 (Bogotá-norte) - Switch2: 60 hosts (PC4, PC5)

- LAN2 (Bogotá-norte) - Switch3: 10 hosts (PC6, PC7)

WAN links (Router interconnections): 5. Router0 ↔ Router1 (top link) 6. Router0 ↔ Router1 (bottom link - if separate)

Total subnets needed: 6 (4 LANs + 2 WAN links)

Step 1: Calculate minimum IP range needed

Order by size (VLSM):

- LAN1 (Bogotá-norte) - Switch2: 60 hosts → need /26 (62 hosts)

- LAN1 (Bogotá-sur) - Switch0: 30 hosts → need /27 (30 hosts)

- LAN2 (Bogotá-sur) - Switch1: 20 hosts → need /27 (30 hosts)

- LAN2 (Bogotá-norte) - Switch3: 10 hosts → need /28 (14 hosts)

- WAN Link 1: 2 hosts → need /30 (2 hosts)

- WAN Link 2: 2 hosts → need /30 (2 hosts)

Total addresses needed: 62 + 30 + 30 + 14 + 2 + 2 = 140 usable addresses

With overhead: approximately 200-256 addresses

Answer to Question 1: Minimum IP range to request from ISP

Recommended: Request a /24 (Class C network)

Why?

- Provides 256 total addresses (254 usable)

- Sufficient for all current requirements (140 hosts)

- Allows room for growth

- Standard allocation size from ISPs

Minimum acceptable: /25

- Provides 128 addresses (126 usable)

- Tight fit but technically sufficient

- Less room for growth

Best answer: Request one /24 network (e.g., 192.168.1.0/24 or any public Class C range)

Question 2: Perform the subnetting

Let's use 192.168.1.0/24 as the assigned range from ISP.

COMPLETE SUBNETTING SOLUTION:

Subnet allocation (ordered largest to smallest):

1. LAN1 Bogotá-norte (Switch2) - 60 hosts

- Network: 192.168.1.0/26

- Mask: 255.255.255.192

- First host: 192.168.1.1

- Last host: 192.168.1.62

- Broadcast: 192.168.1.63

- Usable hosts: 62

- Binary (4th octet): 00000000

- Devices: PC4, PC5

2. LAN1 Bogotá-sur (Switch0) - 30 hosts

- Network: 192.168.1.64/27

- Mask: 255.255.255.224

- First host: 192.168.1.65

- Last host: 192.168.1.94

- Broadcast: 192.168.1.95

- Usable hosts: 30

- Binary (4th octet): 01000000

- Devices: PC0, PC1

3. LAN2 Bogotá-sur (Switch1) - 20 hosts

- Network: 192.168.1.96/27

- Mask: 255.255.255.224

- First host: 192.168.1.97

- Last host: 192.168.1.126

- Broadcast: 192.168.1.127

- Usable hosts: 30

- Binary (4th octet): 01100000

- Devices: PC2, PC3

4. LAN2 Bogotá-norte (Switch3) - 10 hosts

- Network: 192.168.1.128/28

- Mask: 255.255.255.240

- First host: 192.168.1.129

- Last host: 192.168.1.142

- Broadcast: 192.168.1.143

- Usable hosts: 14

- Binary (4th octet): 10000000

- Devices: PC6, PC7

5. WAN Link 1 (Router0 ↔ Router1 - Top)

- Network: 192.168.1.144/30

- Mask: 255.255.255.252

- First host: 192.168.1.145 (Router0 interface)

- Last host: 192.168.1.146 (Router1 interface)

- Broadcast: 192.168.1.147

- Usable hosts: 2

- Binary (4th octet): 10010000

6. WAN Link 2 (Router0 ↔ Router1 - Bottom, if needed)

- Network: 192.168.1.148/30

- Mask: 255.255.255.252

- First host: 192.168.1.149 (Router0 interface)

- Last host: 192.168.1.150 (Router1 interface)

- Broadcast: 192.168.1.151

- Usable hosts: 2

- Binary (4th octet): 10010100

SUMMARY TABLE:

| Subnet | Location | Network | First IP | Last IP | Broadcast | Mask | /n | Hosts | Binary (last octet) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Switch2 (LAN1-Norte) | 192.168.1.0 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 | 255.255.255.192 | /26 | 62 | 00000000 |
| 2 | Switch0 (LAN1-Sur) | 192.168.1.64 | 192.168.1.65 | 192.168.1.94 | 192.168.1.95 | 255.255.255.224 | /27 | 30 | 01000000 |
| 3 | Switch1 (LAN2-Sur) | 192.168.1.96 | 192.168.1.97 | 192.168.1.126 | 192.168.1.127 | 255.255.255.224 | /27 | 30 | 01100000 |
| 4 | Switch3 (LAN2-Norte) | 192.168.1.128 | 192.168.1.129 | 192.168.1.142 | 192.168.1.143 | 255.255.255.240 | /28 | 14 | 10000000 |
| 5 | WAN Link 1 | 192.168.1.144 | 192.168.1.145 | 192.168.1.146 | 192.168.1.147 | 255.255.255.252 | /30 | 2 | 10010000 |
| 6 | WAN Link 2 | 192.168.1.148 | 192.168.1.149 | 192.168.1.150 | 192.168.1.151 | 255.255.255.252 | /30 | 2 | 10010100 |

IP Assignment Example:

Router Interfaces:

Router0:

- Interface to Switch0: 192.168.1.65

- Interface to Switch1: 192.168.1.97

- Interface to Router1 (top): 192.168.1.145

- Interface to Router1 (bottom): 192.168.1.149

Router1:

- Interface to Switch2: 192.168.1.1

- Interface to Switch3: 192.168.1.129

- Interface to Router0 (top): 192.168.1.146

- Interface to Router0 (bottom): 192.168.1.150

FINAL ANSWERS:

1. ¿Cuál es el mínimo rango IP que debe pedir a su ISP?

Answer: Un rango /24 (e.g., 192.168.1.0/24 o cualquier red Clase C pública)

Justification:

- Necesita mínimo 140 direcciones útiles

- /24 proporciona 254 direcciones útiles

- Permite crecimiento futuro

- Es el tamaño estándar más eficiente

2. Realice el subnetting respectivo

✅ Completed above with full VLSM allocation table

Space used: 192.168.1.0 - 192.168.1.151
Space remaining: 192.168.1.152 - 192.168.1.255 (available for future expansion)

- Dada la red del dibujo, distribuya la red 10.23.160.0/20

Las redes LAN requieren 60, 270, 130 y 380 host.

Indique, para cada subred, ID de red, Broadcast, máscara, Gateway y número de equipos en el rango.

Problem Analysis:

Given:

- Network: 10.23.160.0/20

- Available space: 2^12 = 4,096 addresses

LAN Requirements:

- LAN 1: 60 hosts

- LAN 2: 270 hosts

- LAN 3: 130 hosts

- LAN 4: 380 hosts

WAN Links (Router interconnections):

- Router0 ↔ Router1

- Router0 ↔ Router2

- Router1 ↔ Router3

- Router2 ↔ Router3

Total subnets needed: 8 (4 LANs + 4 WAN links)

Step 1: Understand the /20 network

Network: 10.23.160.0/20

Binary breakdown:

10.23.10100000.00000000  (10.23.160.0)

10.23.10101111.11111111  (10.23.175.255)

Range: 10.23.160.0 - 10.23.175.255

- Third octet range: 160-175 (binary: 10100000 to 10101111)

Step 2: Calculate subnet sizes (VLSM)

Order by size (largest first):

- LAN 4: 380 hosts → need 2^9 - 2 = 510 hosts → /23 ✓

- LAN 2: 270 hosts → need 2^9 - 2 = 510 hosts → /23 ✓

- LAN 3: 130 hosts → need 2^8 - 2 = 254 hosts → /24 ✓

- LAN 1: 60 hosts → need 2^6 - 2 = 62 hosts → /26 ✓

- WAN 1-4: 2 hosts each → /30 ✓ (×4)

Step 3: VLSM Subnet Allocation

1. LAN 4 (Router1) - 380 hosts

- Network ID: 10.23.160.0/23

- Mask: 255.255.254.0

- First host: 10.23.160.1

- Last host: 10.23.161.254

- Broadcast: 10.23.161.255

- Gateway: 10.23.160.1 (Router1 interface)

- Usable hosts: 510

- Binary (3rd octet): 10100000 - 10100001

2. LAN 2 (Router2) - 270 hosts

- Network ID: 10.23.162.0/23

- Mask: 255.255.254.0

- First host: 10.23.162.1

- Last host: 10.23.163.254

- Broadcast: 10.23.163.255

- Gateway: 10.23.162.1 (Router2 interface)

- Usable hosts: 510

- Binary (3rd octet): 10100010 - 10100011

3. LAN 3 (Router3) - 130 hosts

- Network ID: 10.23.164.0/24

- Mask: 255.255.255.0

- First host: 10.23.164.1

- Last host: 10.23.164.254

- Broadcast: 10.23.164.255

- Gateway: 10.23.164.1 (Router3 interface)

- Usable hosts: 254

- Binary (3rd octet): 10100100

4. LAN 1 (Router0) - 60 hosts

- Network ID: 10.23.165.0/26

- Mask: 255.255.255.192

- First host: 10.23.165.1

- Last host: 10.23.165.62

- Broadcast: 10.23.165.63

- Gateway: 10.23.165.1 (Router0 interface)

- Usable hosts: 62

- Binary (4th octet): 00000000

5. WAN Link 1: Router0 ↔ Router1

- Network ID: 10.23.165.64/30

- Mask: 255.255.255.252

- Router0 interface: 10.23.165.65

- Router1 interface: 10.23.165.66

- Broadcast: 10.23.165.67

- Usable hosts: 2

- Binary (4th octet): 01000000

6. WAN Link 2: Router0 ↔ Router2

- Network ID: 10.23.165.68/30

- Mask: 255.255.255.252

- Router0 interface: 10.23.165.69

- Router2 interface: 10.23.165.70

- Broadcast: 10.23.165.71

- Usable hosts: 2

- Binary (4th octet): 01000100

7. WAN Link 3: Router1 ↔ Router3

- Network ID: 10.23.165.72/30

- Mask: 255.255.255.252

- Router1 interface: 10.23.165.73

- Router3 interface: 10.23.165.74

- Broadcast: 10.23.165.75

- Usable hosts: 2

- Binary (4th octet): 01001000

8. WAN Link 4: Router2 ↔ Router3

- Network ID: 10.23.165.76/30

- Mask: 255.255.255.252

- Router2 interface: 10.23.165.77

- Router3 interface: 10.23.165.78

- Broadcast: 10.23.165.79

- Usable hosts: 2

- Binary (4th octet): 01001100

COMPLETE SOLUTION TABLE:

| Subred | Ubicación | ID de Red | Broadcast | Máscara | Gateway | Equipos en rango | Binary (last octet) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LAN 4 | Router1 (PC1) | 10.23.160.0/23 | 10.23.161.255 | 255.255.254.0 | 10.23.160.1 | 510 (380 needed) | 10100000.00000000 |
| LAN 2 | Router2 (PC2) | 10.23.162.0/23 | 10.23.163.255 | 255.255.254.0 | 10.23.162.1 | 510 (270 needed) | 10100010.00000000 |
| LAN 3 | Router3 (PC3) | 10.23.164.0/24 | 10.23.164.255 | 255.255.255.0 | 10.23.164.1 | 254 (130 needed) | 10100100.00000000 |
| LAN 1 | Router0 (PC0) | 10.23.165.0/26 | 10.23.165.63 | 255.255.255.192 | 10.23.165.1 | 62 (60 needed) | 10100101.00000000 |
| WAN 1 | R0↔R1 | 10.23.165.64/30 | 10.23.165.67 | 255.255.255.252 | - | 2 | 10100101.01000000 |
| WAN 2 | R0↔R2 | 10.23.165.68/30 | 10.23.165.71 | 255.255.255.252 | - | 2 | 10100101.01000100 |
| WAN 3 | R1↔R3 | 10.23.165.72/30 | 10.23.165.75 | 255.255.255.252 | - | 2 | 10100101.01001000 |
| WAN 4 | R2↔R3 | 10.23.165.76/30 | 10.23.165.79 | 255.255.255.252 | - | 2 | 10100101.01001100 |

Detailed Network Information:

LAN Networks:

| LAN | Router | Hosts | Network | Range | Gateway | PCs |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Router0 | 60 | 10.23.165.0/26 | 10.23.165.1 - 10.23.165.62 | 10.23.165.1 | PC0 |
| 2 | Router2 | 270 | 10.23.162.0/23 | 10.23.162.1 - 10.23.163.254 | 10.23.162.1 | PC2 |
| 3 | Router3 | 130 | 10.23.164.0/24 | 10.23.164.1 - 10.23.164.254 | 10.23.164.1 | PC3 |
| 4 | Router1 | 380 | 10.23.160.0/23 | 10.23.160.1 - 10.23.161.254 | 10.23.160.1 | PC1 |

WAN Links:

| Link | Routers | Network | Interface 1 | Interface 2 |
| --- | --- | --- | --- | --- |
| 1 | R0↔R1 | 10.23.165.64/30 | R0: 10.23.165.65 | R1: 10.23.165.66 |
| 2 | R0↔R2 | 10.23.165.68/30 | R0: 10.23.165.69 | R2: 10.23.165.70 |
| 3 | R1↔R3 | 10.23.165.72/30 | R1: 10.23.165.73 | R3: 10.23.165.74 |
| 4 | R2↔R3 | 10.23.165.76/30 | R2: 10.23.165.77 | R3: 10.23.165.78 |

Space Usage:

- Used: 10.23.160.0 - 10.23.165.79

- Remaining: 10.23.165.80 - 10.23.175.255 (available for future expansion)

- Efficiency: Very good with VLSM

Verification:

✅ All LANs have sufficient capacity:

- LAN 4: 380 needed, 510 available

- LAN 2: 270 needed, 510 available

- LAN 3: 130 needed, 254 available

- LAN 1: 60 needed, 62 available

✅ All subnets within 10.23.160.0/20 range ✅ No overlapping subnets ✅ Efficient VLSM allocation

- Partiendo de la siguiente red:

Diseñe el direccionamiento IP de tal manera que permita la conexión de los hosts de cada una de las redes según el siguiente cuadro. La red dada por el ISP a la empresa dependerá de la necesidad de la misma, así que usted debe indicar la mejor máscara a utilizar. El identificador de red es ISP es 42.16.0.0/20

| Ciudad | No. Equipos |
| --- | --- |
| Bogota_clientes | 290 |
| Bogota_servidores | 50 |
| Cali | 60 |
| Pereira | 80 |
| Medellín | 100 |
| Cartagena | 150 |
| Barranquilla | 270 |
| Bucaramanga | 150 |
| Villavicencio | 90 |

Given:

- ISP Network: 42.16.0.0/20

- Available space: 2^12 = 4,096 addresses (4,094 usable)

City Requirements:

| Ciudad | Equipos | Order |
| --- | --- | --- |
| Bogotá_clientes | 290 | 1st |
| Barranquilla | 270 | 2nd |
| Cartagena | 150 | 3rd |
| Bucaramanga | 150 | 4th |
| Medellín | 100 | 5th |
| Villavicencio | 90 | 6th |
| Pereira | 80 | 7th |
| Cali | 60 | 8th |
| Bogotá_servidores | 50 | 9th |

Total hosts: 290 + 270 + 150 + 150 + 100 + 90 + 80 + 60 + 50 = 1,240 hosts

Additional subnets needed:

- Router WAN links (between routers)

- Estimated 10-15 WAN links based on topology

Step 1: Understand the /20 network

Network: 42.16.0.0/20

Binary:

42.16.00000000.00000000  (42.16.0.0)

42.16.00001111.11111111  (42.16.15.255)

Range: 42.16.0.0 - 42.16.15.255

- Third octet: 0-15 (binary: 00000000 to 00001111)

Step 2: VLSM Subnet Sizing

Ordered by size (largest first):

- Bogotá_clientes: 290 → 2^9 - 2 = 510 → /23 ✓

- Barranquilla: 270 → 2^9 - 2 = 510 → /23 ✓

- Cartagena: 150 → 2^8 - 2 = 254 → /24 ✓

- Bucaramanga: 150 → 2^8 - 2 = 254 → /24 ✓

- Medellín: 100 → 2^7 - 2 = 126 → /25 ✓

- Villavicencio: 90 → 2^7 - 2 = 126 → /25 ✓

- Pereira: 80 → 2^7 - 2 = 126 → /25 ✓

- Cali: 60 → 2^6 - 2 = 62 → /26 ✓

- Bogotá_servidores: 50 → 2^6 - 2 = 62 → /26 ✓

- WAN Links: 2 each → /30 (multiple)

COMPLETE SUBNETTING SOLUTION:

1. Bogotá_clientes (Switch7 + Switch8) - 290 hosts

- Network ID: 42.16.0.0/23

- Mask: 255.255.254.0

- First host: 42.16.0.1

- Last host: 42.16.1.254

- Broadcast: 42.16.1.255

- Gateway: 42.16.0.1 (Router Bogotá)

- Usable hosts: 510

- Devices: PC14, PC15, Switch7, Switch8

- Binary (3rd octet): 00000000 - 00000001

2. Barranquilla (Switch5) - 270 hosts

- Network ID: 42.16.2.0/23

- Mask: 255.255.254.0

- First host: 42.16.2.1

- Last host: 42.16.3.254

- Broadcast: 42.16.3.255

- Gateway: 42.16.2.1 (Router Barranquilla)

- Usable hosts: 510

- Devices: PC8, PC9, Switch5

- Binary (3rd octet): 00000010 - 00000011

3. Cartagena (Switch6) - 150 hosts

- Network ID: 42.16.4.0/24

- Mask: 255.255.255.0

- First host: 42.16.4.1

- Last host: 42.16.4.254

- Broadcast: 42.16.4.255

- Gateway: 42.16.4.1 (Router Cartagena)

- Usable hosts: 254

- Devices: PC6, PC7, Switch6

- Binary (3rd octet): 00000100

4. Bucaramanga (Switch4) - 150 hosts

- Network ID: 42.16.5.0/24

- Mask: 255.255.255.0

- First host: 42.16.5.1

- Last host: 42.16.5.254

- Broadcast: 42.16.5.255

- Gateway: 42.16.5.1 (Router Bucaramanga)

- Usable hosts: 254

- Devices: PC10, PC11, Switch4

- Binary (3rd octet): 00000101

5. Medellín (Switch0) - 100 hosts

- Network ID: 42.16.6.0/25

- Mask: 255.255.255.128

- First host: 42.16.6.1

- Last host: 42.16.6.126

- Broadcast: 42.16.6.127

- Gateway: 42.16.6.1 (Router Medellín)

- Usable hosts: 126

- Devices: PC4, PC5, Switch0

- Binary (4th octet): 00000000

6. Villavicencio (Switch3) - 90 hosts

- Network ID: 42.16.6.128/25

- Mask: 255.255.255.128

- First host: 42.16.6.129

- Last host: 42.16.6.254

- Broadcast: 42.16.6.255

- Gateway: 42.16.6.129 (Router Villavicencio)

- Usable hosts: 126

- Devices: PC12, PC13, Switch3

- Binary (4th octet): 10000000

7. Pereira (Router Pereira) - 80 hosts

- Network ID: 42.16.7.0/25

- Mask: 255.255.255.128

- First host: 42.16.7.1

- Last host: 42.16.7.126

- Broadcast: 42.16.7.127

- Gateway: 42.16.7.1 (Router Pereira)

- Usable hosts: 126

- Binary (4th octet): 00000000

8. Cali (Switch2) - 60 hosts

- Network ID: 42.16.7.128/26

- Mask: 255.255.255.192

- First host: 42.16.7.129

- Last host: 42.16.7.190

- Broadcast: 42.16.7.191

- Gateway: 42.16.7.129 (Router Cali)

- Usable hosts: 62

- Devices: PC0, PC1, Switch2

- Binary (4th octet): 10000000

9. Bogotá_servidores (Switch7) - 50 hosts

- Network ID: 42.16.7.192/26

- Mask: 255.255.255.192

- First host: 42.16.7.193

- Last host: 42.16.7.254

- Broadcast: 42.16.7.255

- Gateway: 42.16.7.193 (Router Bogotá)

- Usable hosts: 62

- Devices: ServerDNS, ServerWeb, ServerMailEst1, ServerMailEst2

- Binary (4th octet): 11000000

WAN Links (/30 networks):

Starting from 42.16.8.0:

| Link | Routers | Network | Interface 1 | Interface 2 | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 1 | Bogotá↔Pereira | 42.16.8.0/30 | 42.16.8.1 | 42.16.8.2 | 42.16.8.3 |
| 2 | Bogotá↔Barranquilla | 42.16.8.4/30 | 42.16.8.5 | 42.16.8.6 | 42.16.8.7 |
| 3 | Bogotá↔Bucaramanga | 42.16.8.8/30 | 42.16.8.9 | 42.16.8.10 | 42.16.8.11 |
| 4 | Pereira↔Medellín | 42.16.8.12/30 | 42.16.8.13 | 42.16.8.14 | 42.16.8.15 |
| 5 | Pereira↔Cali | 42.16.8.16/30 | 42.16.8.17 | 42.16.8.18 | 42.16.8.19 |
| 6 | Cartagena↔Medellín | 42.16.8.20/30 | 42.16.8.21 | 42.16.8.22 | 42.16.8.23 |
| 7 | Cartagena↔Barranquilla | 42.16.8.24/30 | 42.16.8.25 | 42.16.8.26 | 42.16.8.27 |
| 8 | Barranquilla↔Bucaramanga | 42.16.8.28/30 | 42.16.8.29 | 42.16.8.30 | 42.16.8.31 |
| 9 | Bucaramanga↔Villavicencio | 42.16.8.32/30 | 42.16.8.33 | 42.16.8.34 | 42.16.8.35 |
| 10 | Bogotá↔Villavicencio | 42.16.8.36/30 | 42.16.8.37 | 42.16.8.38 | 42.16.8.39 |

SUMMARY TABLE - LAN NETWORKS:

| Ciudad | Equipos | Network ID | Broadcast | Máscara | Gateway | Hosts | Switch/Devices |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bogotá_clientes | 290 | 42.16.0.0/23 | 42.16.1.255 | 255.255.254.0 | 42.16.0.1 | 510 | Switch7, Switch8, PC14-15 |
| Barranquilla | 270 | 42.16.2.0/23 | 42.16.3.255 | 255.255.254.0 | 42.16.2.1 | 510 | Switch5, PC8-9 |
| Cartagena | 150 | 42.16.4.0/24 | 42.16.4.255 | 255.255.255.0 | 42.16.4.1 | 254 | Switch6, PC6-7 |
| Bucaramanga | 150 | 42.16.5.0/24 | 42.16.5.255 | 255.255.255.0 | 42.16.5.1 | 254 | Switch4, PC10-11 |
| Medellín | 100 | 42.16.6.0/25 | 42.16.6.127 | 255.255.255.128 | 42.16.6.1 | 126 | Switch0, PC4-5 |
| Villavicencio | 90 | 42.16.6.128/25 | 42.16.6.255 | 255.255.255.128 | 42.16.6.129 | 126 | Switch3, PC12-13 |
| Pereira | 80 | 42.16.7.0/25 | 42.16.7.127 | 255.255.255.128 | 42.16.7.1 | 126 | Connected to router |
| Cali | 60 | 42.16.7.128/26 | 42.16.7.191 | 255.255.255.192 | 42.16.7.129 | 62 | Switch2, PC0-1 |
| Bogotá_servidores | 50 | 42.16.7.192/26 | 42.16.7.255 | 255.255.255.192 | 42.16.7.193 | 62 | Switch7, Servers |

Space Usage:

- LANs used: 42.16.0.0 - 42.16.7.255

- WANs used: 42.16.8.0 - 42.16.8.39

- Remaining: 42.16.8.40 - 42.16.15.255 (available for expansion)

Verification:

✅ All cities have sufficient capacity ✅ Efficient VLSM allocation ✅ All subnets within 42.16.0.0/20 ✅ No overlapping addresses ✅ Room for future growth

- Partiendo de la siguiente red:

Diseñe el direccionamiento IP de tal manera que permita la conexión de los hosts de cada una de las redes según el siguiente cuadro. La red dada por el ISP a la empresa dependerá de la necesidad de la misma, así que usted debe indicar la mejor máscara a utilizar. La red de arranque es la 20.135.128.0/18

| RED | No. Host |
| --- | --- |
| Houston | 60 |
| Bogotá | 490 |
| Santiago | 600 |
| Brasilia | 500 |
| Murcia | 220 |
| Londres | 50 |
| Berlín | 480 |
| Estocolmo | 550 |

Omita los óvalos presentes en el dibujo y en las redes inalámbricas suponga que se puede usar una red del rango dado y que en cada red WiFi se requieren 30 equipos.

## Problem Analysis:

### Given:

- ISP Network: 20.135.128.0/18

- Available space: 2^14 = 16,384 addresses (16,382 usable)

### Requirements:

Wired LANs:

| RED | Hosts |
| --- | --- |
| Santiago | 600 |
| Estocolmo | 550 |
| Brasilia | 500 |
| Bogotá | 490 |
| Berlín | 480 |
| Murcia | 220 |
| Houston | 60 |
| Londres | 50 |

Wireless Networks (WiFi):

- 6 wireless networks × 30 hosts each

WAN Links:

- Multiple router interconnections (approximately 12-15 links)

Total wired hosts: 600 + 550 + 500 + 490 + 480 + 220 + 60 + 50 = 2,950 hosts
Total WiFi hosts: 6 × 30 = 180 hosts
Total: ~3,130 hosts + WAN links

## Step 1: Understand the /18 network

Network: 20.135.128.0/18

Binary:

20.135.10000000.00000000  (20.135.128.0)

20.135.10111111.11111111  (20.135.191.255)

Range: 20.135.128.0 - 20.135.191.255

- Third octet: 128-191 (binary: 10000000 to 10111111)

## Step 2: VLSM Subnet Sizing

### Ordered by size (largest first):

Wired LANs:

- Santiago: 600 → 2^10 - 2 = 1,022 → /22 ✓

- Estocolmo: 550 → 2^10 - 2 = 1,022 → /22 ✓

- Brasilia: 500 → 2^9 - 2 = 510 → /23 ✓

- Bogotá: 490 → 2^9 - 2 = 510 → /23 ✓

- Berlín: 480 → 2^9 - 2 = 510 → /23 ✓

- Murcia: 220 → 2^8 - 2 = 254 → /24 ✓

- Houston: 60 → 2^6 - 2 = 62 → /26 ✓

- Londres: 50 → 2^6 - 2 = 62 → /26 ✓

Wireless Networks: 9-14. WiFi (6 networks): 30 each → 2^5 - 2 = 30 → /27 ✓

WAN Links: 15+. Router links: 2 each → /30 ✓

## COMPLETE SUBNETTING SOLUTION:

## WIRED LAN NETWORKS:

### 1. Santiago - 600 hosts

- Network ID: 20.135.128.0/22

- Mask: 255.255.252.0

- First host: 20.135.128.1

- Last host: 20.135.131.254

- Broadcast: 20.135.131.255

- Gateway: 20.135.128.1 (Router Chile)

- Usable hosts: 1,022

- Devices: Switch Santiago, PC3, PC4

- Binary (3rd octet): 10000000 - 10000011

### 2. Estocolmo - 550 hosts

- Network ID: 20.135.132.0/22

- Mask: 255.255.252.0

- First host: 20.135.132.1

- Last host: 20.135.135.254

- Broadcast: 20.135.135.255

- Gateway: 20.135.132.1 (Router Suecia)

- Usable hosts: 1,022

- Devices: Switch Estocolmo, PC9, PC10, PC11

- Binary (3rd octet): 10000100 - 10000111

### 3. Brasilia - 500 hosts

- Network ID: 20.135.136.0/23

- Mask: 255.255.254.0

- First host: 20.135.136.1

- Last host: 20.135.137.254

- Broadcast: 20.135.137.255

- Gateway: 20.135.136.1 (Router Brasil)

- Usable hosts: 510

- Devices: Switch Brasilia, PC5, PostgreSQL_Server1

- Binary (3rd octet): 10001000 - 10001001

### 4. Bogotá - 490 hosts

- Network ID: 20.135.138.0/23

- Mask: 255.255.254.0

- First host: 20.135.138.1

- Last host: 20.135.139.254

- Broadcast: 20.135.139.255

- Gateway: 20.135.138.1 (Router Colombia)

- Usable hosts: 510

- Devices: Switch Bogotá, PC1, ApacheWebServer

- Binary (3rd octet): 10001010 - 10001011

### 5. Berlín - 480 hosts

- Network ID: 20.135.140.0/23

- Mask: 255.255.254.0

- First host: 20.135.140.1

- Last host: 20.135.141.254

- Broadcast: 20.135.141.255

- Gateway: 20.135.140.1 (Router Alemania)

- Usable hosts: 510

- Devices: Switch Berlín, PC9, PC10, PostgreSQL_Server2

- Binary (3rd octet): 10001100 – 10001101

### 6. Murcia - 220 hosts

- Network ID: 20.135.142.0/24

- Mask: 255.255.255.0

- First host: 20.135.142.1

- Last host: 20.135.142.254

- Broadcast: 20.135.142.255

- Gateway: 20.135.142.1 (Router España)

- Usable hosts: 254

- Devices: Switch Murcia, PC6, PC7

- Binary (3rd octet): 10001110

### 7. Houston - 60 hosts

- Network ID: 20.135.143.0/26

- Mask: 255.255.255.192

- First host: 20.135.143.1

- Last host: 20.135.143.62

- Broadcast: 20.135.143.63

- Gateway: 20.135.143.1 (Router USA)

- Usable hosts: 62

- Devices: Switch Houston, PC0, PC1, WRT300N_CeremesNA

- Binary (4th octet): 00000000

### 8. Londres - 50 hosts

- Network ID: 20.135.143.64/26

- Mask: 255.255.255.192

- First host: 20.135.143.65

- Last host: 20.135.143.126

- Broadcast: 20.135.143.127

- Gateway: 20.135.143.65 (Router Inglaterra)

- Usable hosts: 62

- Devices: Switch Londres, PC2, PC8, IIS_Server

- Binary (4th octet): 01000000

## WIRELESS NETWORKS (WiFi):

### 9. WiFi Houston (WRT300N_CeremesNA) - 30 hosts

- Network ID: 20.135.143.128/27

- Mask: 255.255.255.224

- First host: 20.135.143.129

- Last host: 20.135.143.158

- Broadcast: 20.135.143.159

- Gateway: 20.135.143.129

- Usable hosts: 30

- Binary (4th octet): 10000000

### 10. WiFi Londres (Smartphone Estudiante4) - 30 hosts

- Network ID: 20.135.143.160/27

- Mask: 255.255.255.224

- First host: 20.135.143.161

- Last host: 20.135.143.190

- Broadcast: 20.135.143.191

- Gateway: 20.135.143.161

- Usable hosts: 30

- Binary (4th octet): 10100000

### 11. WiFi Estocolmo (WRT300N_Geremesilez) - 30 hosts

- Network ID: 20.135.143.192/27

- Mask: 255.255.255.224

- First host: 20.135.143.193

- Last host: 20.135.143.222

- Broadcast: 20.135.143.223

- Gateway: 20.135.143.193

- Usable hosts: 30

- Binary (4th octet): 11000000

### 12. WiFi Murcia (Smartphone Estudiante2) - 30 hosts

- Network ID: 20.135.143.224/27

- Mask: 255.255.255.224

- First host: 20.135.143.225

- Last host: 20.135.143.254

- Broadcast: 20.135.143.255

- Gateway: 20.135.143.225

- Usable hosts: 30

- Binary (4th octet): 11100000

### 13. WiFi Santiago (WRT300N_GeremesLA) - 30 hosts

- Network ID: 20.135.144.0/27

- Mask: 255.255.255.224

- First host: 20.135.144.1

- Last host: 20.135.144.30

- Broadcast: 20.135.144.31

- Gateway: 20.135.144.1

- Usable hosts: 30

- Binary (4th octet): 00000000

### 14. WiFi Brasilia (Smartphone Estudiante3) - 30 hosts

- Network ID: 20.135.144.32/27

- Mask: 255.255.255.224

- First host: 20.135.144.33

- Last host: 20.135.144.62

- Broadcast: 20.135.144.63

- Gateway: 20.135.144.33

- Usable hosts: 30

- Binary (4th octet): 00100000

## WAN LINKS (/30):

Starting from 20.135.144.64:

| Link | Routers | Network | Interface 1 | Interface 2 | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 1 | USA↔Inglaterra | 20.135.144.64/30 | 20.135.144.65 | 20.135.144.66 | 20.135.144.67 |
| 2 | Inglaterra↔Alemania | 20.135.144.68/30 | 20.135.144.69 | 20.135.144.70 | 20.135.144.71 |
| 3 | USA↔Colombia | 20.135.144.72/30 | 20.135.144.73 | 20.135.144.74 | 20.135.144.75 |
| 4 | Colombia↔Chile | 20.135.144.76/30 | 20.135.144.77 | 20.135.144.78 | 20.135.144.79 |
| 5 | Chile↔Brasil | 20.135.144.80/30 | 20.135.144.81 | 20.135.144.82 | 20.135.144.83 |
| 6 | Brasil↔España | 20.135.144.84/30 | 20.135.144.85 | 20.135.144.86 | 20.135.144.87 |
| 7 | España↔Alemania | 20.135.144.88/30 | 20.135.144.89 | 20.135.144.90 | 20.135.144.91 |
| 8 | Alemania↔Suecia | 20.135.144.92/30 | 20.135.144.93 | 20.135.144.94 | 20.135.144.95 |

(Additional WAN links can be allocated from 20.135.144.96 onwards as needed)

## SUMMARY TABLE:

| RED | Hosts | Network ID | Broadcast | Máscara | Gateway | Usable | Tipo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Santiago | 600 | 20.135.128.0/22 | 20.135.131.255 | 255.255.252.0 | 20.135.128.1 | 1,022 | LAN |
| Estocolmo | 550 | 20.135.132.0/22 | 20.135.135.255 | 255.255.252.0 | 20.135.132.1 | 1,022 | LAN |
| Brasilia | 500 | 20.135.136.0/23 | 20.135.137.255 | 255.255.254.0 | 20.135.136.1 | 510 | LAN |
| Bogotá | 490 | 20.135.138.0/23 | 20.135.139.255 | 255.255.254.0 | 20.135.138.1 | 510 | LAN |
| Berlín | 480 | 20.135.140.0/23 | 20.135.141.255 | 255.255.254.0 | 20.135.140.1 | 510 | LAN |
| Murcia | 220 | 20.135.142.0/24 | 20.135.142.255 | 255.255.255.0 | 20.135.142.1 | 254 | LAN |
| Houston | 60 | 20.135.143.0/26 | 20.135.143.63 | 255.255.255.192 | 20.135.143.1 | 62 | LAN |
| Londres | 50 | 20.135.143.64/26 | 20.135.143.127 | 255.255.255.192 | 20.135.143.65 | 62 | LAN |
| WiFi Houston | 30 | 20.135.143.128/27 | 20.135.143.159 | 255.255.255.224 | 20.135.143.129 | 30 | WiFi |
| WiFi Londres | 30 | 20.135.143.160/27 | 20.135.143.191 | 255.255.255.224 | 20.135.143.161 | 30 | WiFi |
| WiFi Estocolmo | 30 | 20.135.143.192/27 | 20.135.143.223 | 255.255.255.224 | 20.135.143.193 | 30 | WiFi |
| WiFi Murcia | 30 | 20.135.143.224/27 | 20.135.143.255 | 255.255.255.224 | 20.135.143.225 | 30 | WiFi |
| WiFi Santiago | 30 | 20.135.144.0/27 | 20.135.144.31 | 255.255.255.224 | 20.135.144.1 | 30 | WiFi |
| WiFi Brasilia | 30 | 20.135.144.32/27 | 20.135.144.63 | 255.255.255.224 | 20.135.144.33 | 30 | WiFi |

## Space Usage:

- LANs: 20.135.128.0 - 20.135.143.127

- WiFi: 20.135.143.128 - 20.135.144.63

- WANs: 20.135.144.64 - 20.135.144.95

- Remaining: 20.135.144.96 - 20.135.191.255 (for future expansion)

## Verification:

✅ All cities have sufficient capacity
✅ All WiFi networks have 30 hosts
✅ Efficient VLSM allocation
✅ All within 20.135.128.0/18
✅ No overlapping subnets
✅ Plenty of room for growth

- La empresa de lácteos "Mi vaquita" es una empresa dedicada a la producción de alimentos derivados de la leche, como son quesos y yogures. Su ISP le asignó la dirección IP 220.120.32.0/22 para las redes de sus puntos: fábrica, ventas, desarrollo de nuevos productos y administración y la interconexión entre ellas. Sus operaciones están distribuidads así: fábrica, ventas y desarrollo de nuevos productos en Sopó, Ventas en Zipaquirá, Chía, Mosquera y La Calera y ventas y administración en Bogotá. En dichas sedes tiene redes locales con 180, 30, 55, 14, 43 y 130 computadores respectivamente. Realice el subnetting para la empresa.

Problem Analysis:

Given:

- ISP Network: 220.120.32.0/22

- Available space: 2^10 = 1,024 addresses (1,022 usable)

Locations and Requirements:

Based on the problem:

| Sede | Computadores |
| --- | --- |
| Sede 1 | 180 |
| Sede 2 | 130 |
| Sede 3 | 55 |
| Sede 4 | 43 |
| Sede 5 | 30 |
| Sede 6 | 14 |

Total hosts: 180 + 130 + 55 + 43 + 30 + 14 = 452 computers

Distribution by location:

Sopó (3 locations):

- Fábrica

- Ventas

- Desarrollo de nuevos productos

Other locations:

- Zipaquirá (Ventas)

- Chía (Ventas)

- Mosquera (Ventas)

- La Calera (Ventas)

Bogotá (2 locations):

- Ventas

- Administración

Total: 9 locations + WAN links between them

Step 1: Understand the /22 network

Network: 220.120.32.0/22

Binary:

220.120.00100000.00000000  (220.120.32.0)

220.120.00100011.11111111  (220.120.35.255)

Range: 220.120.32.0 - 220.120.35.255

- Third octet: 32-35 (binary: 00100000 to 00100011)

Step 2: VLSM Subnet Sizing

Ordered by size (largest first):

- Sede 1 (180 hosts): → 2^8 - 2 = 254 → /24 ✓

- Sede 2 (130 hosts): → 2^8 - 2 = 254 → /24 ✓

- Sede 3 (55 hosts): → 2^6 - 2 = 62 → /26 ✓

- Sede 4 (43 hosts): → 2^6 - 2 = 62 → /26 ✓

- Sede 5 (30 hosts): → 2^5 - 2 = 30 → /27 ✓

- Sede 6 (14 hosts): → 2^4 - 2 = 14 → /28 ✓

- WAN Links: 2 each → /30 ✓ (multiple)

Step 3: Assign to Specific Locations

Let me map the requirements to specific locations:

Largest networks (180, 130):

- Fábrica Sopó: 180 hosts

- Ventas/Administración Bogotá: 130 hosts

Medium networks (55, 43):

- Desarrollo Sopó: 55 hosts

- Ventas Zipaquirá: 43 hosts

Small networks (30, 14):

- Ventas Chía: 30 hosts

- Ventas Mosquera: 14 hosts

Additional (if needed):

- Ventas La Calera: Can use a /28 or /29

COMPLETE SUBNETTING SOLUTION:

1. Fábrica - Sopó (180 hosts)

- Network ID: 220.120.32.0/24

- Mask: 255.255.255.0

- First host: 220.120.32.1

- Last host: 220.120.32.254

- Broadcast: 220.120.32.255

- Gateway: 220.120.32.1

- Usable hosts: 254

- Purpose: Producción de lácteos

- Binary (3rd octet): 00100000

2. Ventas/Administración - Bogotá (130 hosts)

- Network ID: 220.120.33.0/24

- Mask: 255.255.255.0

- First host: 220.120.33.1

- Last host: 220.120.33.254

- Broadcast: 220.120.33.255

- Gateway: 220.120.33.1

- Usable hosts: 254

- Purpose: Ventas y administración central

- Binary (3rd octet): 00100001

3. Desarrollo de Nuevos Productos - Sopó (55 hosts)

- Network ID: 220.120.34.0/26

- Mask: 255.255.255.192

- First host: 220.120.34.1

- Last host: 220.120.34.62

- Broadcast: 220.120.34.63

- Gateway: 220.120.34.1

- Usable hosts: 62

- Purpose: I+D de productos lácteos

- Binary (4th octet): 00000000

4. Ventas - Zipaquirá (43 hosts)

- Network ID: 220.120.34.64/26

- Mask: 255.255.255.192

- First host: 220.120.34.65

- Last host: 220.120.34.126

- Broadcast: 220.120.34.127

- Gateway: 220.120.34.65

- Usable hosts: 62

- Purpose: Punto de ventas Zipaquirá

- Binary (4th octet): 01000000

5. Ventas - Chía (30 hosts)

- Network ID: 220.120.34.128/27

- Mask: 255.255.255.224

- First host: 220.120.34.129

- Last host: 220.120.34.158

- Broadcast: 220.120.34.159

- Gateway: 220.120.34.129

- Usable hosts: 30

- Purpose: Punto de ventas Chía

- Binary (4th octet): 10000000

6. Ventas - Mosquera (14 hosts)

- Network ID: 220.120.34.160/28

- Mask: 255.255.255.240

- First host: 220.120.34.161

- Last host: 220.120.34.174

- Broadcast: 220.120.34.175

- Gateway: 220.120.34.161

- Usable hosts: 14

- Purpose: Punto de ventas Mosquera

- Binary (4th octet): 10100000

7. Ventas - La Calera (hosts adicionales si necesario)

- Network ID: 220.120.34.176/28

- Mask: 255.255.255.240

- First host: 220.120.34.177

- Last host: 220.120.34.190

- Broadcast: 220.120.34.191

- Gateway: 220.120.34.177

- Usable hosts: 14

- Purpose: Punto de ventas La Calera

- Binary (4th octet): 10110000

WAN LINKS (Router Interconnections):

Starting from 220.120.34.192:

| Link | Conexión | Network | Interface 1 | Interface 2 | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 1 | Fábrica Sopó ↔ Desarrollo Sopó | 220.120.34.192/30 | 220.120.34.193 | 220.120.34.194 | 220.120.34.195 |
| 2 | Fábrica Sopó ↔ Bogotá | 220.120.34.196/30 | 220.120.34.197 | 220.120.34.198 | 220.120.34.199 |
| 3 | Bogotá ↔ Zipaquirá | 220.120.34.200/30 | 220.120.34.201 | 220.120.34.202 | 220.120.34.203 |
| 4 | Bogotá ↔ Chía | 220.120.34.204/30 | 220.120.34.205 | 220.120.34.206 | 220.120.34.207 |
| 5 | Bogotá ↔ Mosquera | 220.120.34.208/30 | 220.120.34.209 | 220.120.34.210 | 220.120.34.211 |
| 6 | Bogotá ↔ La Calera | 220.120.34.212/30 | 220.120.34.213 | 220.120.34.214 | 220.120.34.215 |
| 7 | Sopó ↔ Zipaquirá (if direct) | 220.120.34.216/30 | 220.120.34.217 | 220.120.34.218 | 220.120.34.219 |

SUMMARY TABLE:

| Sede | Ubicación | Hosts | Network ID | Broadcast | Máscara | Gateway | Usable | Propósito |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Fábrica Sopó | 180 | 220.120.32.0/24 | 220.120.32.255 | 255.255.255.0 | 220.120.32.1 | 254 | Producción |
| 2 | Bogotá | 130 | 220.120.33.0/24 | 220.120.33.255 | 255.255.255.0 | 220.120.33.1 | 254 | Ventas/Admin |
| 3 | Desarrollo Sopó | 55 | 220.120.34.0/26 | 220.120.34.63 | 255.255.255.192 | 220.120.34.1 | 62 | I+D |
| 4 | Zipaquirá | 43 | 220.120.34.64/26 | 220.120.34.127 | 255.255.255.192 | 220.120.34.65 | 62 | Ventas |
| 5 | Chía | 30 | 220.120.34.128/27 | 220.120.34.159 | 255.255.255.224 | 220.120.34.129 | 30 | Ventas |
| 6 | Mosquera | 14 | 220.120.34.160/28 | 220.120.34.175 | 255.255.255.240 | 220.120.34.161 | 14 | Ventas |
| 7 | La Calera | 14 | 220.120.34.176/28 | 220.120.34.191 | 255.255.255.240 | 220.120.34.177 | 14 | Ventas |

Network Diagram:

[Fábrica Sopó]

220.120.32.0/24

180 hosts

|

[Router Sopó]

/     \

/       \

[Desarrollo]         [WAN] ←→ [Router Bogotá]

220.120.34.0/26              220.120.33.0/24

55 hosts                     130 hosts (Ventas/Admin)

|

┌─────────────┼─────────────┐

|             |             |

[Zipaquirá]    [Chía]      [Mosquera]  [La Calera]

34.64/26       34.128/27   34.160/28   34.176/28

43 hosts       30 hosts    14 hosts    14 hosts

Space Usage:

- LANs: 220.120.32.0 - 220.120.34.191

- WANs: 220.120.34.192 - 220.120.34.219

- Remaining: 220.120.34.220 - 220.120.35.255 (available for future expansion)

Verification:

✅ All locations have sufficient capacity:

- Fábrica: 180 needed, 254 available

- Bogotá: 130 needed, 254 available

- Desarrollo: 55 needed, 62 available

- Zipaquirá: 43 needed, 62 available

- Chía: 30 needed, 30 available

- Mosquera: 14 needed, 14 available

- La Calera: 14 available

✅ Efficient VLSM allocation
✅ All within 220.120.32.0/22
✅ No overlapping subnets
✅ Room for growth (~300 addresses remaining)

Best Practices Applied:

- ✅ VLSM for efficient IP usage

- ✅ Hierarchical design (Central hubs: Sopó and Bogotá)

- ✅ /30 for WAN links (point-to-point)

- ✅ Scalability for future expansion

- ✅ Logical grouping by function and location

- La Escuela, para su red, requiere direccionar varias redes. Cuenta con la red 10.0.0.0/8, distribuya de la mejor manera las direcciones asignadas entre las redes de tal manera que se puedan tener direcciones IP para todos los hosts en cada subred. Las subredes y cantidades de equipos en cada red se presentan en la tabla siguiente:

| Subred | No. Equipos |
| --- | --- |
| Profesores | 500 |
| Estudiantes | 2500 |
| Personal administrativo | 150 |
| Lab. Informática | 300 |
| Telefonía | 400 |
| Cámaras | 250 |
| Red inalámbrica estudiantes | 3500 |
| Red inalámbrica empleados | 800 |
| Red inalámbrica de invitados | 100 |
| Red de administración de TI | 150 |

Problem Analysis:

Given:

- Network: 10.0.0.0/8 (Class A private network)

- Available space: 2^24 = 16,777,216 addresses

Requirements:

| Subred | No. Equipos | Order |
| --- | --- | --- |
| Red inalámbrica estudiantes | 3,500 | 1st |
| Estudiantes | 2,500 | 2nd |
| Red inalámbrica empleados | 800 | 3rd |
| Profesores | 500 | 4th |
| Telefonía | 400 | 5th |
| Lab. Informática | 300 | 6th |
| Cámaras | 250 | 7th |
| Personal administrativo | 150 | 8th |
| Red de administración de TI | 150 | 9th |
| Red inalámbrica de invitados | 100 | 10th |

Total hosts: 3,500 + 2,500 + 800 + 500 + 400 + 300 + 250 + 150 + 150 + 100 = 8,650 hosts

Step 1: VLSM Subnet Sizing

Ordered by size (largest first):

- Red inalámbrica estudiantes: 3,500 → 2^12 - 2 = 4,094 → /20 ✓

- Estudiantes: 2,500 → 2^12 - 2 = 4,094 → /20 ✓

- Red inalámbrica empleados: 800 → 2^10 - 2 = 1,022 → /22 ✓

- Profesores: 500 → 2^9 - 2 = 510 → /23 ✓

- Telefonía: 400 → 2^9 - 2 = 510 → /23 ✓

- Lab. Informática: 300 → 2^9 - 2 = 510 → /23 ✓

- Cámaras: 250 → 2^8 - 2 = 254 → /24 ✓

- Personal administrativo: 150 → 2^8 - 2 = 254 → /24 ✓

- Red de administración de TI: 150 → 2^8 - 2 = 254 → /24 ✓

- Red inalámbrica de invitados: 100 → 2^7 - 2 = 126 → /25 ✓

COMPLETE SUBNETTING SOLUTION:

1. Red Inalámbrica Estudiantes - 3,500 hosts

- Network ID: 10.0.0.0/20

- Mask: 255.255.240.0

- First host: 10.0.0.1

- Last host: 10.0.15.254

- Broadcast: 10.0.15.255

- Gateway: 10.0.0.1

- Usable hosts: 4,094

- Purpose: WiFi para estudiantes

- Binary (2nd-3rd octet): 00000000.00000000 - 00000000.00001111

2. Estudiantes (Red Cableada) - 2,500 hosts

- Network ID: 10.0.16.0/20

- Mask: 255.255.240.0

- First host: 10.0.16.1

- Last host: 10.0.31.254

- Broadcast: 10.0.31.255

- Gateway: 10.0.16.1

- Usable hosts: 4,094

- Purpose: Red cableada estudiantes

- Binary (2nd-3rd octet): 00000000.00010000 - 00000000.00011111

3. Red Inalámbrica Empleados - 800 hosts

- Network ID: 10.0.32.0/22

- Mask: 255.255.252.0

- First host: 10.0.32.1

- Last host: 10.0.35.254

- Broadcast: 10.0.35.255

- Gateway: 10.0.32.1

- Usable hosts: 1,022

- Purpose: WiFi para profesores y personal

- Binary (3rd octet): 00100000 - 00100011

4. Profesores - 500 hosts

- Network ID: 10.0.36.0/23

- Mask: 255.255.254.0

- First host: 10.0.36.1

- Last host: 10.0.37.254

- Broadcast: 10.0.37.255

- Gateway: 10.0.36.1

- Usable hosts: 510

- Purpose: Red cableada profesores

- Binary (3rd octet): 00100100 - 00100101

5. Telefonía IP - 400 hosts

- Network ID: 10.0.38.0/23

- Mask: 255.255.254.0

- First host: 10.0.38.1

- Last host: 10.0.39.254

- Broadcast: 10.0.39.255

- Gateway: 10.0.38.1

- Usable hosts: 510

- Purpose: Sistema de telefonía VoIP

- Binary (3rd octet): 00100110 - 00100111

6. Laboratorio de Informática - 300 hosts

- Network ID: 10.0.40.0/23

- Mask: 255.255.254.0

- First host: 10.0.40.1

- Last host: 10.0.41.254

- Broadcast: 10.0.41.255

- Gateway: 10.0.40.1

- Usable hosts: 510

- Purpose: Computadores de laboratorio

- Binary (3rd octet): 00101000 - 00101001

7. Cámaras de Seguridad - 250 hosts

- Network ID: 10.0.42.0/24

- Mask: 255.255.255.0

- First host: 10.0.42.1

- Last host: 10.0.42.254

- Broadcast: 10.0.42.255

- Gateway: 10.0.42.1

- Usable hosts: 254

- Purpose: Sistema de videovigilancia IP

- Binary (3rd octet): 00101010

8. Personal Administrativo - 150 hosts

- Network ID: 10.0.43.0/24

- Mask: 255.255.255.0

- First host: 10.0.43.1

- Last host: 10.0.43.254

- Broadcast: 10.0.43.255

- Gateway: 10.0.43.1

- Usable hosts: 254

- Purpose: Red administrativa

- Binary (3rd octet): 00101011

9. Red de Administración de TI - 150 hosts

- Network ID: 10.0.44.0/24

- Mask: 255.255.255.0

- First host: 10.0.44.1

- Last host: 10.0.44.254

- Broadcast: 10.0.44.255

- Gateway: 10.0.44.1

- Usable hosts: 254

- Purpose: Gestión de infraestructura IT

- Binary (3rd octet): 00101100

10. Red Inalámbrica de Invitados - 100 hosts

- Network ID: 10.0.45.0/25

- Mask: 255.255.255.128

- First host: 10.0.45.1

- Last host: 10.0.45.126

- Broadcast: 10.0.45.127

- Gateway: 10.0.45.1

- Usable hosts: 126

- Purpose: WiFi para visitantes (aislada)

- Binary (4th octet): 00000000

SUMMARY TABLE:

| Subred | Equipos | Network ID | Broadcast | Máscara | Gateway | Usable | /n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Red WiFi Estudiantes | 3,500 | 10.0.0.0 | 10.0.15.255 | 255.255.240.0 | 10.0.0.1 | 4,094 | /20 |
| Estudiantes Cableada | 2,500 | 10.0.16.0 | 10.0.31.255 | 255.255.240.0 | 10.0.16.1 | 4,094 | /20 |
| Red WiFi Empleados | 800 | 10.0.32.0 | 10.0.35.255 | 255.255.252.0 | 10.0.32.1 | 1,022 | /22 |
| Profesores | 500 | 10.0.36.0 | 10.0.37.255 | 255.255.254.0 | 10.0.36.1 | 510 | /23 |
| Telefonía IP | 400 | 10.0.38.0 | 10.0.39.255 | 255.255.254.0 | 10.0.38.1 | 510 | /23 |
| Lab. Informática | 300 | 10.0.40.0 | 10.0.41.255 | 255.255.254.0 | 10.0.40.1 | 510 | /23 |
| Cámaras | 250 | 10.0.42.0 | 10.0.42.255 | 255.255.255.0 | 10.0.42.1 | 254 | /24 |
| Personal Admin | 150 | 10.0.43.0 | 10.0.43.255 | 255.255.255.0 | 10.0.43.1 | 254 | /24 |
| Admin de TI | 150 | 10.0.44.0 | 10.0.44.255 | 255.255.255.0 | 10.0.44.1 | 254 | /24 |
| WiFi Invitados | 100 | 10.0.45.0 | 10.0.45.127 | 255.255.255.128 | 10.0.45.1 | 126 | /25 |

Network Segmentation by Function:

Student Networks:

10.0.0.0/20   - WiFi Estudiantes (3,500)

10.0.16.0/20  - Cableada Estudiantes (2,500)

Staff Networks:

10.0.32.0/22  - WiFi Empleados (800)

10.0.36.0/23  - Profesores (500)

10.0.43.0/24  - Personal Administrativo (150)

Infrastructure Networks:

10.0.38.0/23  - Telefonía IP (400)

10.0.40.0/23  - Lab. Informática (300)

10.0.42.0/24  - Cámaras (250)

10.0.44.0/24  - Administración TI (150)

Guest Network:

10.0.45.0/25  - WiFi Invitados (100) - Isolated

Security Recommendations:

VLANs and Segmentation:

| VLAN ID | Network | Purpose | Security Level |
| --- | --- | --- | --- |
| 10 | 10.0.0.0/20 | WiFi Estudiantes | Medium |
| 20 | 10.0.16.0/20 | Estudiantes Cableada | Medium |
| 30 | 10.0.32.0/22 | WiFi Empleados | High |
| 40 | 10.0.36.0/23 | Profesores | High |
| 50 | 10.0.38.0/23 | Telefonía (VoIP) | Critical |
| 60 | 10.0.40.0/23 | Lab. Informática | Medium |
| 70 | 10.0.42.0/24 | Cámaras | Critical |
| 80 | 10.0.43.0/24 | Personal Admin | High |
| 90 | 10.0.44.0/24 | Admin TI | Critical |
| 99 | 10.0.45.0/25 | WiFi Invitados | Low (Isolated) |

Firewall Rules Recommendations:

- ✅ WiFi Invitados: Internet only, no access to internal networks

- ✅ Cámaras: Access only from Admin TI network

- ✅ Telefonía: QoS priority, restricted access

- ✅ Admin TI: Full access to all networks for management

- ✅ Estudiantes: Limited access to administrative networks

Space Usage:

- Used: 10.0.0.0 - 10.0.45.127

- Remaining: 10.0.45.128 - 10.255.255.255 (16+ million addresses for future expansion!)

DHCP Pools Recommendation:

| Network | DHCP Pool Start | DHCP Pool End | Static Range |
| --- | --- | --- | --- |
| WiFi Estudiantes | 10.0.0.100 | 10.0.15.250 | 10.0.0.1 - 10.0.0.99 |
| Estudiantes | 10.0.16.100 | 10.0.31.250 | 10.0.16.1 - 10.0.16.99 |
| WiFi Empleados | 10.0.32.100 | 10.0.35.250 | 10.0.32.1 - 10.0.32.99 |
| Profesores | 10.0.36.100 | 10.0.37.250 | 10.0.36.1 - 10.0.36.99 |
| Telefonía | 10.0.38.100 | 10.0.39.250 | 10.0.38.1 - 10.0.38.99 |
| Lab. Informática | 10.0.40.10 | 10.0.41.250 | 10.0.40.1 - 10.0.40.9 |
| Cámaras | N/A (Static only) | N/A | 10.0.42.1 - 10.0.42.254 |
| Personal Admin | 10.0.43.50 | 10.0.43.250 | 10.0.43.1 - 10.0.43.49 |
| Admin TI | 10.0.44.10 | 10.0.44.100 | 10.0.44.1 - 10.0.44.9 |
| WiFi Invitados | 10.0.45.10 | 10.0.45.126 | 10.0.45.1 - 10.0.45.9 |

Verification:

✅ All networks have sufficient capacity with room to grow
✅ Efficient VLSM allocation
✅ Logical segmentation by function and security level
✅ Isolated guest network
✅ Scalable design for future expansion
✅ QoS-ready for VoIP (Telefonía)
✅ Security camera network isolated